Proposal Title:
Evaluating Surface Water Management System Using ICPR and BMP’s Train Under New Criteria of Evaluation for Environmental Permits: Case Study in SFWMD.

Proposal PI

Name: Higinio L Hernandez E.I.
Department of Civil Engineering, U.A. Whitaker College of Engineering, Florida Gulf Coast University, Fort Myers, Florida, hlhernandezarenc5660@eagle.fgcu.edu

Program Information:
EGN 6457 – CRN 83614 Research Methods

Proposal Submission Date: December 7, 2025

Proposal Start Date: January 7, 2026

Proposal End Date: May 2, 2026

Budget:
None

Abstract

Hurricane Ian caused more than five feet of flooding at the Church of the Ascension site in The Town of  Fort Myers Beach, damaging the existing stormwater management system. At the same time, Florida’s updated stormwater rules, including SB 712 and the new SFWMD treatment criteria, now require stronger flood control and nutrient reduction performance. This project evaluates and redesigns the 14.61-acre property, including its 13.05-acre stormwater system, to meet these new standards and improve long-term resilience.

The study uses ICPR modeling to assess hydrologic and hydraulic performance under 10 Year-1 Day, 25 Year-3 Day, and 100 Year-3 Day storms events, and BMP Trains to quantify water quality treatment and nutrient reductions. Several design scenarios, including green infrastructure options, will be integrated to identify the most effective system. The expected results include reduced flooding, improved water quality, and a set of practical design recommendations that can guide the redevelopment of this site under the updated SFWMD treatment criteria and serve as a model for other projects in South Florida.

# 1.0 Project Summary

Background. Fort Myers Beach is highly vulnerable to flooding and storm impacts due to its low-lying coastal location. The Church of the Ascension, located on Estero Boulevard, was destroyed by Hurricane Ian on September 28, 2022, when the site experienced over five feet of flooding. The storm also damaged the existing stormwater management system, highlighted need for updated Surface Stormwater Management System, (SWMS). Rebuilding the site presents an opportunity to restore functionality and integrate a resilient, sustainable stormwater system that can better manage flood risks and improve water quality.

Recent updates to Florida’s stormwater regulations provide a framework for designing such resilient systems. The Clean Waterways Act (SB 712, 2020), followed by the Technical Advisory Committee’s recommendations and subsequent rulemaking, established new operational, maintenance, and stormwater treatment criteria. These rules, ratified through SB 7040 (2024) and effective June 2024 and December 2025 for inspectors and treatment criteria, set stricter nutrient removal standards and flood resilience requirements.

Objective. The primary goal of this research is to evaluate the performance of the proposed SWMS under the new SFWMD criteria and propose alternative green solutions to improve system resilience, water quality, and regulatory compliance. The study will assess hydraulic performance, flood mitigation, and nutrient reduction potential, while modeling scenarios for storm rainfall events. The outcome will inform design recommendations for a practical and sustainable stormwater system.

Methods. We will use ICPR modeling to simulate hydrology, storage, and discharge under existing and proposed SWMS configurations. Design storms include 10-Year -1 Day, 25-year-3 Day, and 100-Year- 3 Day events, with back-to-back storms evaluated to ensure basin recovery.  Nutrient loading will be analyzed using BMP Trains methodology, routing runoff through dry retention, wet detention, and green option basins in series. Green alternatives including stormwater harvesting or vegetated buffers  will be assessed for performance in nutrient reduction and flood mitigation. Hydrologic  modeling will identify the most effective design for compliance with updated  SFWMD stormwater criteria.

Outputs. The project will produce detailed hydraulic and water quality performance metrics, including basin storage levels, discharge rates, and nutrient removal efficiency. Performance comparisons of the traditional system with green alternatives, along with technical design recommendations for SWMS improvements. Results will be compiled in a comprehensive technical report, supported by ICPR modeling outputs, BMPs Train nutrient analysis, and scenario  assessments of stormwater management strategies.

Impacts. This research will provide guidance for redesigned and improving stormwater management at the site location while complying with updated regulatory requirements. By evaluating green alternatives, the study promotes sustainable design approaches that enhance flood resilience and water quality. Findings will inform local planners and engineers on effective SWMS design for coastal sites. Additionally, the project will serve as a model for integrating conventional stormwater systems with specific green infrastructure solutions under updated Florida stormwater criteria, supporting broader environmental protection and public safety objectives.

# 1.0 Introduction

Coastal regions such as Fort Myers Beach are increasingly susceptible to storm rainfall events, sea level rise, flooding, and high pollution level on water. The site is along Estero Boulevard which exemplifies this vulnerability, having been destroyed by Hurricane Ian on September 28, 2022. The event caused over five feet of flooding, which damaged the existing SWMS that can’t properly       convey and store stormwater runoff, putting the site at risk of future flooding.

Recent regulatory updates, including SB 712 (Clean Waterways Act, 2020) and subsequent rulemaking, establish new stormwater treatment criteria that became effective on December 28, 2025. These regulations enhanced water quality treatment standards. Evaluating and redesigning the site’s SWMS under these updated criteria is critical to ensure resilience against storm rainfall events, improve water quality, and achieve compliance with district evaluation standards.

This project proposes systematic assessment and redesigning the site’s stormwater system for redevelopment using hydrologic and hydraulic modeling, nutrient load analysis, and green infrastructure evaluation. The study will assess site hydrology, water quality, and nutrient retention while exploring sustainable stormwater management alternatives. The project area, encompassing 14.61 acres, will undergo substantial reconstruction, including demolition of damaged buildings and parking areas, and construction of a new church building and associated infrastructure. The proposed SWMS, covering approximately 13.05 acres, will include the new church, parking, dry retention areas, and lakes designed to optimize stormwater conveyance, water quality, and flood protection.
The project site is located at 6025 Estero Boulevard, Fort Myers Beach, FL 33931, on the north side of Estero ROW at the intersection of Estero Boulevard and Mound Road. It is situated in Lee County, Florida, within Section 33, Township 46 South, Range 24 East.

Figure 1.Study area location. (Lee County Appraiser, 2025)

The existing SWMS was previously modified under Permit No. 36-103211-P and included a church, multiple buildings, parking lots, two lakes, and dry retention areas. Historically, stormwater flowed toward on-site detention areas before being conveyed to the existing lakes via drainage pipes and swales. The existing system discharges off-site into the North Lake, which is tidally influenced and ultimately drains into Estero Bay. Surrounding properties are residential and multi-family, with no off-site inflows affecting the site.

The SWMS will be modified with additional dry retention areas and expansion of the existing west lake. Stormwater will be conveyed via sheet flow or inlets to the dry retention and lakes in series to provide water quality treatment and nutrient load reduction. Stormwater will discharge into the North Lake through the existing control weir, ultimately flowing north via a drainage ditch to Estero Bay. All elevations and design calculations will reference NAVD-88.

1.1 Literature Review and Research Gaps

Urban stormwater management has evolved significantly over the past decades, but major challenges remain, particularly in coastal and redevelopment contexts. Traditional stormwater systems, often referred to as gray infrastructure, such as pipes, ditches, dry and wet detention ponds, and infiltration basins, have historically been designed to rapidly convey runoff away from urban areas (Tsegaye,2019). Harper and Baker (2007) conducted a comprehensive review of stormwater management systems in Florida, assessing pollutant removal efficiencies for dry retention, wet detention, dry detention with filtration, and off-line retention/detention ponds. They found that removal efficiencies vary widely depending on design, site conditions, and monitoring methodology, with total phosphorus (TP) removal ranging from approximately 61% to 95% and nitrogen species achieving up to 90% removal under ideal conditions. However, limitations included sparse monitoring events and previous criteria of evaluation for environmental permits.

More recently, the transition from gray to green infrastructure has gained attention. Tsegaye et al. (2019) introduced the Gray-to-Green article, which integrates green infrastructure BMPs within urban forests to manage stormwater at or near its source. Green infrastructure provides multiple co-benefits: reducing runoff, enhancing infiltration, supporting urban vegetation, improving water quality, and generating environmental, economic, and social benefits. The article emphasizes planning and communication across disciplines, illustrating how green BMPs can be integrated into redevelopment and existing urban landscapes, which are particularly relevant for coastal and highly urbanized sites.

Regulatory frameworks also play a critical role. Florida Statutes Title XXVIII, Chapter 373, Section 373.4131, outlines updated statewide environmental resource permitting rules, including post-development nutrient load reduction targets and additional requirements for sub-watersheds containing Outstanding Florida Waters  or impaired waters. Applicants are required to demonstrate through modeling and calculations that stormwater treatment systems meet these performance standards. These regulations underscore the need for site specific analysis.

Nagendra Prasad et al. (2022) provides a global review of sustainable and resilient urban stormwater management practices, highlighting the need to consider climate change, land-use patterns, hydrologic alterations, treatment technologies, and societal factors in urban planning. Integrated modeling approaches, including LID, BMPs, Water Sensitive Urban Design, and Sponge City principles, are recommended to achieve sustainable and resilient outcomes. Despite these advances, significant gaps remain in linking model predictions to on-the-ground performance for redeveloped or coastal sites.

Collectively, the literature indicates several critical research gaps:

- Limited data on post-redevelopment BMP performance in coastal areas subject to tidal influence, storm surge, and sea-level rise.

- Sparse comparative studies evaluating traditional gray infrastructure against integrated green infrastructure strategies in real world urban settings.

- Insufficient modeling frameworks that combine hydrologic, hydraulic, water quality, and nutrient load assessments under updated regulatory requirements.

This project aims to fill these gaps by evaluating the proposed stormwater management system under the new Florida regulatory framework. The narrowly focused research problem is: How effective is the proposed SWMS (dry + wet in series) in reducing nitrogen and phosphorus under the updated criteria of evaluation, improve flood resilience, enhance water quality? Do green alternatives (stormwater harvesting or vegetated buffers) provide measurable credit for nutrient reduction? The study proposes to: (1) assess site hydrology, existing system performance, and flooding impacts; (2) model stormwater conveyance and water quality treatment using design storms events and ICPR and BMPs Train modeling; (3) explore green infrastructure alternatives for improved nutrient reduction; and (4) develop a compliant and sustainable stormwater management system for the project area, which  including new buildings, parking, retention and wet pond areas.

The research is feasible within the project timeline and resources, leveraging existing hydrologic and hydraulic modeling design, publicly available data, and established regulatory guidance. The outcomes will provide actionable design recommendations for the site and serve as a replicable framework for other coastal properties in South Florida. This study also aligns with current sustainability goals, integrating regulatory compliance, stormwater performance, and environmental protection into a realistic, research driven solution.

2.0 Methods

To evaluate, document, and redesign the SWMS, this study will employ a mixed-method approach that combines hydrologic and hydraulic modeling, nutrient and water quality analysis, and green infrastructure assessment. The approach is designed to ensure clarity, reliability, reproducibility, and alignment with the updated SFWMD stormwater criteria effective December 28, 2025.

2.1 Data Collection and Site Assessment
We will compile detailed site data including topography, existing infrastructure, soil types, land use/cover, and design storm events. Key sources include Lee County LiDAR GIS, Lee County Soil Survey, aerial imagery, and existing permit documentation (Permit No. 36-103211-P). Stormwater elevations and pre-development drainage patterns will be collected from field inspections and historical records. The site has experienced over five feet of flooding during Hurricane Ian, compromising existing SWMS and motivating a full hydrologic and hydraulic evaluation. This step ensures that all subsequent modeling reflects realistic conditions and site-specific characteristics.

2.2 Hydrologic and Hydraulic Modeling
The study will use ICPR modeling to simulate stormwater routing, storage, and discharge under design storm criteria. The modeling will evaluate the SWMS under multiple design events: 10-year, 1-day storm (7.2 in), 25-year, 3-day storm (11.2 in), and 100-year, 3-day storm (15.1 in). The ICPR model will also simulate back-to-back storms to test basin recovery, ensuring perimeter berms and retention areas are adequate for design storm scenarios. Control elevations, discharge rates, and tailwater conditions will be defined based on NAVD-88 datum and tidal influences from the North Lake and Estero Bay. Model reliability is supported by historical validation against observed storm events and existing ERP permit data.

2.3 Water Quality and Nutrient Loading Analysis

The stormwater quality evaluation will follow SFWMD Applicant Handbook Volume I, Section 8, along with the updated nutrient treatment criteria that take effect on December 28, 2025. To determine the required treatment level, the project’s location will be compared against Section 8.3, which assigns nutrient reduction targets based on a site’s proximity to sensitive waters. In general, standard sites require reductions of about 55 percent for total nitrogen (TN) and 80 percent for total phosphorus (TP). Sites upstream of Outstanding Florida Waters or impaired waters require stricter treatment, typically 80 to 90 percent, with added conditions such as meeting pre-development loads or providing a measurable net improvement. The most stringent category applies to areas upstream of both impaired and Outstanding Florida Waters, where reductions of about 95 percent are required. Redevelopment areas fall under a separate set of thresholds, ranging from roughly 45 to 80 percent depending on their environmental sensitivity.

After the applicable category is identified, a pre- and post-development pollutant load analysis will be completed. The final nutrient removal percentage used for the project will be whichever value is higher: the required reduction from Section 8 or the percentage found through the pre/post analysis. This ensures the redesigned system meets regulatory expectations while reflecting actual site performance. The study will also evaluate dry retention and wet detention areas in series to estimate treatment efficiency and identify opportunities to boost nutrient removal through added green infrastructure.

Table 1. Updated SFWMD Criteria for Evaluation.

| Project Location | Nutrient Reduction Requirements | Additional Criteria |
| --- | --- | --- |
| All Sites (Minimum) | - 80% reduction TP - 55% reduction TN | Or post ≤ pre |
| Upstream of OFW | - 90% reduction in TP - 80% reduction in TN | Or post ≤ pre |
| Upstream of Impaired Waters | - 80% reduction in TP - 80% reduction in TN | And post ≤ pre plus net improvement for the pollutant of concern |
| Upstream Impaired + OFW | - 95% reduction in TP - 95% reduction in TN | And post ≤ pre plus net improvement |
| Redevelopment | - 80% reduction in TP - 45% reduction in TN | N/A |
| Redevelopment + OFW | - 90% reduction in TP - 60% reduction in TN | N/A |
| Redevelopment + Impaired | - 80% reduction in TP - 45% reduction in TN | And net improvement for the pollutant of concern |

2.4 Green Infrastructure and Design Alternatives
Alternative design scenarios will be evaluated for their ability to reduce nutrient loads and improve flood resilience. Strategies include stormwater harvesting  and vegetated buffers. Each alternative will be assessed through BMPs Train simulations to determine its effect on water quality and pollution reduction. Optimization will focus on meeting the new SFWMD requirements while improving environmental performance and regulatory compliance. Finally, the alternatives will be compared through a cost-effective analysis to determine whether they are practical and realistic to include in future development.

2.5 Data Analysis and Interpretation
Results from ICPR modeling and BMPs Train nutrient analysis will be analyzed to compare pre- and post-development performance, peak elevations, allowable discharge rate, flood mitigation, and nutrient removal efficiency. Conservative assumptions will be applied to account for uncertainty in rainfall patterns, tailwater conditions, and soil characteristics. Data analysis procedures are fully documented to allow step-by-step reproducibility by other researchers.

2.6 Ethical Considerations
Ethical considerations for this study include environmental protection by minimizing impacts, maintaining water quality, and preserving nearby ecosystems in SFWMD. Ethical considerations focus on environmental and engineering  responsibility, including accurate representation of site conditions, responsible use of modeling software, and transparent reporting of results. All data supporting the project is publicly accessible and analysis will be shared in accordance with professional and academic standards.

2.7 Anticipate  Findings
Based on preliminary modeling and site assessments, the study anticipates that the redesigned SWMS will significantly reduce on-site flooding, improve water quality, and achieve nutrient load reductions in accordance with updated SFWMD criteria. Specifically, the ICPR model simulations are expected to demonstrate that the proposed dry retention and wet detention areas, combined with green infrastructure elements such as stormwater harvesting  and vegetated buffers, will manage storm events up to the 10 Year-1 Day, 25 Year-3 Day, and 100 Year-3 Day  while maintaining allowable discharge rates. Additionally, nutrient loading analyses using BMP Trains are anticipated to show measurable reductions in nitrogen and phosphorus concentrations compared to pre-development conditions. The findings will provide evidence for flood mitigation and water quality improvement strategies.

2.8 Deliverables

- A comprehensive hydrologic and hydraulic analysis report, including ICPR model inputs, outputs, and performance for multiple design storms.

- A water quality and nutrient loading assessment, including BMPs Train model inputs, outputs demonstrating the effectiveness of the SWMS in reducing pollutant loads for the proposed land use cover.

- Design recommendations and alternative scenarios for green infrastructure integration to improve SWMS performance and compliance with the new SFWMD criteria.

- Detailed construction and maintenance guidelines for the redevelopment aligned with the updated stormwater treatment criteria.

- A public post at FGCU event outlining the effectiveness of SWMS for site and regulatory compliance.

2.9 Outputs
The project outputs will include:

- Quantitative data showing peak stage elevations, storage volume, and discharge performance under 10 Year-1 Day, 25 Year-3 Day, and 100 Year-3 Day design storms.

- Modeled reductions in nutrient loads (nitrogen and phosphorus) from the SWMS, demonstrating compliance with water quality treatment targets.

- Performance comparisons across multiple green infrastructure alternatives, highlighting the most effective solutions for water treatment criteria.

- GIS-based maps, System Diagram, and Unit Hydrograph chart visualizations of stormwater in/out flow, stage elevations, storage, and treatment areas for use in planning, permitting, and informative.

2.10 Outcomes
Short-term outcomes include:

- A validated, site-specific SWMS design capable of managing severe storm events and reducing flood risk on the subject property.

- Implementation ready designs for dry retention, wet detention, and green infrastructure features that improve water quality while meeting SFWMD treatment criteria.

- Measurable nutrient load reductions and improved water conveyance that support environmental protection goals for Estero Bay Basin.

2.11 Impacts
Long-term impacts of the project will include:

- Enhanced resilience of the development against future rainfall events, minimizing property damage and flood hazards.

- A model framework for coastal property stormwater management that can be replicated at other sites within Town of Fort Myers Beach and similar coastal communities.

- Improved compliance with Florida’s SB 712 and SB 7040 legislation, including new stormwater treatment criteria.

- Positive contributions to community awareness of sustainable stormwater practices, climate adaptation strategies, and green infrastructure solutions.

2.12 Feasibility
The proposed research is realistic and achievable within the project’s scope and timeline. All required data, including site topography, soil surveys, design storm events, and SFWMD regulatory guidance, are available. ICPR modeling, BMP Trains analysis, and green infrastructure evaluation are well-established, reliable methods that align with the research objectives. The project scope, covering a 13.05-acre SWMS on the 14.61-acre site, ensures manageable analysis and implementation planning. Resource requirements, including software, field data, and professional expertise, are feasible and accessible, ensuring successful completion of all deliverables and outcomes.

## 3.0  Timeline. The contribution of the PI at FGCU.

Table 2. Research project timeline.

| Week | Task | Milestone/Deliverable | Contingency Plan |
| --- | --- | --- | --- |
| 1 | Project kickoff, site assessment, and data collection planning | Completion of detailed project plan and data collection checklist | If site access is delayed, prioritize reviewing permitted data and regulations first. |
| 2 | Collect and process existing data (rainfall, tailwater, soil, site topography) | Preliminary dataset compiled | Use publicly available datasets if site data is incomplete. |
| 3 | Hydrologic modeling setup (ICPR model) | Model setup completed for baseline conditions | Validate with pre-existing model results if issues arise. |
| 4 | Hydraulic modeling of pre-development SWMS | Pre-development scenario results | Run sensitivity analysis to address tailwater or rainfall conditions. |
| 5 | Water quality and nutrient loading analysis pre-post development (BMPs Train) | TN and TP reduction estimates for pre-post development scenario | If data gaps exist, apply conservative assumptions per SFWMD criteria. |
| 6 | Green infrastructure alternatives assessment | Preliminary list of feasible alternatives | If some alternatives cannot be assessed due to limits, rank based on literature-based performance. |
| 7 | Modeling post-development scenarios with selected alternatives | Post-development SWMS performance results | Run multiple conservative scenarios to account for uncertainty. |
| 8 | Comparative analysis and cost-effectiveness evaluation for green alternatives | Selection of final alternative(s) | If costs exceed feasibility, consider traditional components. |
| 9 | Draft final report and prepare visuals for submission | Complete draft with figures and tables. Meeting with assigned professor | Allocate extra time for data verification or missing analysis. |
| 10 | Final revisions, review, and submission | Final report submitted | Plan for peer review feedback and last-minute adjustments. |

## 4.0  AI Assistance Statement

AI assistance from ChatGPT-4 was utilized for providing review comments, refining text for  clarity and suggestions to improve the project analysis. AI contributions were verified to ensure accuracy and relevance to the project's objectives. The author has reviewed and edited the output and content of this publication.

5.0 Supplementary Materials: The supporting information and backups used for this Case Study can be find at: https://github.com/HiginioH/rm.git

6.0 Acknowledgments: The author would like to acknowledge the support provided by Oscar Lugo, PE and the research assistance provided by Seneshaw Tsegaye, Ph.D,  Mebrahtom Kebedew, Ph.D, and Ahmed S. Elshall, Ph.D During the preparation of this proposal research.

7.0 Conflicts of Interest: There are no known conflicts of interest associated with this project. All environmental data used in the analysis is publicly available as part of the ERP permit application, and no proprietary or restricted information was utilized.

8.0 References

- Harry H. Harper, David M. Baker, Evaluation of Current Stormwater Design Criteria within the State of Florida. Final Report Florida Department of Environmental Protection,2007.

- Tsegaye, S.; Singleton, T.L.; Koeser, A.K.; Lamb, D.S.; Landry, S.M.; Lu, S.; Barber, J.B.; Hilbert, D.R.; Hamilton, K.O.; Northrop, R.J.; et al. Transitioning from gray to green (G2G) A green infrastructure planning tool for the urban forest. Urban For. Urban Green. 2019, 40, 204–214.Transitioning from gray to green (G2G)—A green infrastructure planning tool for the urban forest - ScienceDirect

- Lakshmi Raghu Nagendra Prasad Rentachintala; M. G. Muni Reddy; Pranab Kumar Mohapatra, Urban stormwater management for sustainable and resilient measures and practices: a review. Water Science & Technology (2022) 85 (4): 1120–1140.

- The Florida Senate. 2025 Florida Statutes. Section 373.4131 Statewide environmental resource permitting rules. Available online: Chapter 373 Section 4131 - 2025 Florida Statutes - The Florida Senate (accessed on 7 December 2025)

- Derek C.S. Burr, Impervious Surface Analysis of the Estero Bay Watershed in Lee County, Florida. Degree of Master of Science, Faculty of the College of Arts and Sciences, Florida Gulf Coast University, 2008.

- Daniel W. Schroeder, Seneshaw Tsegaye, Thomas L. Singleton, Kevin K. Albrecht, GIS- and ICPR-Based Approach to Sustainable Urban Drainage Practices: Case Study of a Development Site in Florida. Water 2022, 14, 1557. https://doi.org/10.3390/w14101557

- Aguiar, C.R.; Nuernberg, J.K.; Leonardi, T.C. Multicriteria GIS-Based Approach in Priority Areas Analysis for Sustainable Urban Drainage Practices: A Case Study of Pato Branco, Brazil. Eng 2020, 1, 96–111. [CrossRef]

- Flood Insurance Rate Maps. Available online: https://msc.fema.gov/portal/home (accessed on 25 November 2025)

- Collier County Land Development Code. Available online: https://library.municode.com/fl/collier_county/codes/code_of_ordinances (accessed s on 25 November 2025).

- South Florida Water Management District. Regulation—Permits. Available online: https://www.sfwmd.gov/doing-business with-us/permits (accessed s on 25 November 2025).

- Applicant’s Handbook Vol. I. Available online: https://www.sfwmd.gov/sites/default/files/documents/swerp_applicants_handbook_vol_i.pdf (accessed on 25 November 2025).

- Applicant’s Handbook Vol. II. Available online: https://www.sfwmd.gov/sites/default/files/documents/archive_swerp_applicants_handbook_vol_ii_20240628.pdf (accessed on 25 November 2025).

- Streamline Technologies Inc. Interconnected Channel and Pond Routing Model. Available online: https://streamline.tech/resources/gis-and-icpr (accessed s on 25 November 2025).

- NOAA Atlas 14 Point Precipitation. Available online: https://hdsc.nws.noaa.gov/pfds/pfds_map_cont.html?bkmrk=fl